Equipment Performance Measurements

For

WAHT

1560 kHz

# Cowpens SC

FCC ID 24482

October 17, 2019

## Measurements taken by

Alan Alsobrook

Alsobrook Electronics

13 Matanzas Cir.

St. Augustine Fl. 32080

(904) 829-8885

## General

These pages document the procedures and results of the measurements required by title 47 section 73.1590, as specified in section 73.44 of the Code of Federal Regulations, to demonstrate compliance with in-band emission requirements, and for harmonic content.

#### Measurement Procedure

## A swept frequency RF spectrum analyzer with a resolution of 30 Hz, using a peak hold of a minimum of 10 minutes was used for these measurements.  The signal was received off air with an untuned shielded loop antenna at a distance of approximately 1-kilometer from the antenna site.  In-band emissions removed from the carrier by more than 40 kHz, as well as checks for harmonic content were made with a precision communications receiver.    All measurements were made during daylight hours for all modes of operation.

#### Equipment list

## Spectrum Analyzer	Anritsu MS2721B

Precision Communications Receiver	Potomac Fim-41

Antenna	AE-LA2

#### Technical Qualifications

All measurements were made by Alan Alsobrook, who is a Certified Senior Radio Engineer #3338 by the SBE, and also holds FCC General Radio Telephone Operators License PG-6-11216.   Mr. Alsobrook has served as a Radio Engineer since 1977.

Alan H Alsobrook

NDA-Day

## Horizontal 5kHz/Div.	Vertical 10db/Div	RBW 30Hz

73.44 (b) Attenuation of emissions below unmodulated carrier.

#### Range		Requirement	Measured

10.2	to	20kHz	-25db	-40db

## Frequency Measurement 1560.001 kHz  Error +1 Hz

##### NDA-Day

## Horizontal 10kHz/Div.	Vertical 10db/Div	RBW 30Hz

73.44 (b) Attenuation of emissions below unmodulated carrier.

#### Range		Requirement	Measured

20	to	30kHz	-35db	-57db

30	to	60kHz	5 + 1db/khz	Compliant

60	to	75kHz	-65db	-83 db

## Beyond 75 kHz, 		The lesser of 80db or

43 + 10 Log (Power in watts)

NDA-Day

## Horizontal 20 kHz/Div.	Vertical 10db/Div	RBW 30Hz

73.44 (b) Attenuation of emissions below unmodulated carrier.

##### Beyond 75 kHz  = 80 dB

## Using the receiver section of the IFR Com 120 b the spectrum was swept from 535 kHz – 5000 kHz, the following readings were obtained.

## In band spurs (535 kHz – 1705 kHz) 	Not detected

2nd harmonic 	3120 kHz	-83 dB

3rd Harmonic	4680 kHz	-84 dB

4th Harmonic	6240 kHz	Not detected

5th Harmonic	7800 kHz	Not detected